MultiCell Layout

Hierarchical Design

Lower level cell → Higher level cell: Maintain hierarchical structure

MUST “instantiate cell” not “copying cell” : press ‘i’ → ‘browse’ to select the cell

Use ‘shift-F’ to peak into the internal layers of the cell

Use ‘ctrl-f’ to hide the cell polygons. – DO NOT FLATTEN YOUR CELL!

MUST add power and ground rail pins to each cell.

Cell Placement

When combining multiple instances, place cells in rows so that the vdd/gnd metals are adjacent to each other.

Power Rail Widening

Combining more cells: increase in the current flowing though the power rails.

Need to ensure power rails can handle the peak current load of all circuits attached to the rail – Difficult to determine.

MUST estimate the “max current though any row of circuits (a row shares power supply rails) and estimate the length of the row → Knowing sheet resistance of metal1 you calculate the max resistance of the power rail (sheet resistance × length/width).

Calculate the maximum voltage drop at max current → Set the height of the power rail so the maximum voltage drop is below 0.1V

Signal Routing

Keep routing within the cell boundary using poly and metal1.

For signals that cannot be routed within the cell boundary, use vertical metal2 to get the signal out of the cell boundary to a horizontal metal1 routing trace that is placed on the top or bottom of the supply rails → Metal2 must be used only in the vertical direction

Connecting Metal1 and Metal2 → use appropriate via contacts – keep width of the metal1 and metal2 (12um)

Try to place all outside routing either on the top or the bottom of your cell but not both.

Connecting Power Rails for Multi-row Layouts

Building a circuit block that uses more than one row of cells, MUST connect the power supply rails (VDD, GND)

1) Virtual Connections (권장)

Labeling each VDD/GND rial (typically within a row of cells) individually and join those using virtual connections → preferred when the power rails will all be physically joined at a later stage in the design.

Checking the option “Joint nets with same names” in the extractor window. If you do not do this, you will get LVS errors for having multiple nets with the same name.

2) Physical connection by polygons

Preferred all of the VDD/GND connections within a cell physically joined together.

Requiring knowledge of higher levels in the layout hierarchy.

If you know the power rails will be joined at a higher level in the design → physical connections (must remove later)

To make single power rails for multi-row cells, use two vertical routes of metal2 (VDD/GND) then add horizontal metal1 and vias.

Edit-in-Place Technique

Important: DO NOT FLATTED YOUR CELLS!

Wanting to immediately see how making a change in an instantiated layout affects the higher level one.

Wanting to quickly make a change to an instantiated layout without having to open a new window editing window.

Using “Edit in Place” command.

1) Select the instance that you wish to edit by left clicking on it

2) From the menu, select Design => Hierarchy => Edit in Place. Alternatively, you can

press “Control-x”.

3) This will put an orange box around the instance which you may now edit.

4) If you want to descend further into the hierarchy, repeat steps 1-4 as necessary.

5) When you are done editing and wish to save the changes, check and save the design.

6) Select Design => Hierarchy => Return to return the editor scope to the top-level layout.

Alternatively, you can press “b”.

7) If you made changes that weren’t saved, Cadence will prompt you for a save. If you don’t

wish to save, make sure to select “No”. This will restore the layout to its original form.